WISCONSIN RACING FORMULA SAE – SPROCKET FABRICATION PROCESS

UNIVERSITY OF WISCONSIN – MADISON, SPRING 2019

GALEN RILEY

JANUARY 23RD, 2019

- START WITH STOCK MATERIAL – 8’’ DIA X 3.0’’ ROUND STOCK (7075 – T6 ALUMINUM).

- SQUARE STOCK USING A LATHE AND TURN DOWN TO 7.6’’ DIAMETER.

- PROCEED TO BORE A HOLE WITH A 1.5’’ DIAMETER.

- USING THE “T-REX” BAND SAW IN THE STUDENT SHOP, PART THE STOCK EXACTLY IN HALF. YOU SHOULD NOW HAVE TWO PIECE OF STOCK WITH A THICKNESS OF ROUGHLY 1.50’’

- RETURN TO THE LATHE AND CUT EACH OF THE TWO PIECES DOWN TO THEIR FINAL THICKNESS OF 1.07’’ ENSURING THAT BOTH FACES ON EACH PIECE OF STOCK IS SQUARE.

- USING THE HAAS (OR A CNC LATHE), CAREFULLY FIXTURE THE PART UTILIZING THE BORED OUT FEATURE AND CUT THE SPROCKET PROFILE GEOMETRY INTO THE STOCK. (SANDING MAY BE REQUIRED).

- USING THE HAAS (OR A CNC LATHE), USE SIMILAR FIXTURING TECHNIQUES FROM STEP 6 TO CUT THE SPROCKET TOOTH TAPER INTO BOTH SIDES OF THE STOCK. (SANDING MAY BE REQUIRED).

- SEND STOCK TO WEASLER TO HAVE THEM WIRE EDM THE DIFFERENTIAL SPLINE GEOMETRY AND THE SPROCKET TOOTH GEOMETRY.

- USING THE HAAS, CUT THE SPOKE GEOMETRY INTO THE SPROCKET.

NOTE: CONSIDER REVERSING STEP 8 AND STEP 9 BASED OFF OF HOW EASY IT IS TO FIXTURE THE PART.